- Support Multitouch Simple User Manual

- Modify your kernel source code refer to drivers\hid files.

- 1 kernel3.0+\drivers\hid\hid-ids.h

- Define Touch VID/PID macro in about line 209.

- 2 Kernel3.0+\drivers\hid\hid-core.c

- Add Touch VID/PID to struct hid_have_special_driver in about line 1378 to 1538.

- 3 Kernel3.0+\drivers\hid\hid-multitouch.c

- Modify the Touch VID/PID at array mt_devices[] in about line 624 to 2054.

- 4 Confirm if the drive have added the hid-multitouch module in kernel config file.

- Run make menuconfig in command terminal to select HID Multitouch panels.

Device Drivers --->

HID Devices --->

Special HID Drivers --->

HID Multitouch panels